Annexure B Part 1 – Carbon Dioxide Specifications (Net Zero North Sea Storage)

- Core specification

| Component | Limit | Units | Notes |
| --- | --- | --- | --- |
| CO2 | ≥96 | mol % |  |
| H2O | ≤50 | ppm mol |  |
| H2 | ≤0.75 | mol % | Note 1, 2 above apply |
| N2 | Combined limit of ≤4 | mol % | Note 1 – Cumulative inerts and non-condensible limit of ≤4mol % (including light hydrocarbons). Note 2 – Additionally for Humber onshore pipeline system the saturation pressure of the mixture shall not exceed 80 barg. |
| Ar |  |  |  |
| Methane |  |  |  |
| Ethane |  |  |  |
| O2 | ≤10 | ppm mol |  |
| SOx (SO, SO₂, SO₃) | ≤10 | ppm mol |  |
| NOx (NO, NO₂) | ≤5 | ppm mol |  |
| H2S | ≤5 | ppm mol |  |
| CO | ≤1000 | Ppm mol | Note 1, 2 above apply |
| Mercury (Hg) | ≤0.0025 | ppm mol |  |
| Glycols | ≤1 | ppm mol |  |
| Amines | ≤1 | ppm mol |  |
| Ammonia | ≤10 | ppm mol |  |
| Formaldehyde | ≤20 | ppm mol |  |
| Acetaldehyde | ≤20 | ppm mol |  |
| Cadmium | ≤0.005 | ppm mol |  |
| Thallium | ≤0.012 | ppm mol |  |
| Methanol | ≤350 | ppm mol | Cumulative methanol and ethanol limit of less than 350 ppm. |
| Ethanol | ≤50 | ppm mol |  |
| Heavy Hca (C3+) | Heavy hydrocarbons shall not shift the dew point below that of pure carbon dioxide e.g. n-C4: ≤0.25mol%, n-C6: ≤250ppm, n-C10: ≤5ppm. |  |  |
| Particulates | Particles shall be ≤5 micron in size and with a maximum occurrence of ≤1 mg/Nm3.v |  |  |

- ADDITIONAL REQUIREMENTS

In addition to the requirements set out in Paragraph ‎1, the following requirements for Teesside Users entering the onshore gathering network must be met with respect to pressure and temperature:

| Parameters | Units | Value | Notes |
| --- | --- | --- | --- |
| Design Conditions |  |  |  |
| Design Pressure | Barg | 44 |  |
| Maximum Design Temperature | °C | 75 |  |
| Minimum Design Temperature | °C | -29 |  |
| Operating Parameters |  |  |  |
| Minimum operating pressure (MinOP) | barg | 15 | Note 1 |
| Maximum operating pressure (MaxOP) | barg | 40 | Note 1 |
| 0.........Maximum operating temperature | °C | 50 | At User’s Delivery Point Note 2 |
| Minimum operating temperature | °C | 15 | At User’s Delivery Point. This is the lowest temperature of the safe operating envelope. Note 2 |

Note 1: The User’s delivery pressure shall be the pressure required to enable the User to deliver CO2 to the Delivery Point (as defined in the Network Code) after taking account of any, and all, back pressure in the T&SCo network at the Delivery Point [“Delivery Pressure”]. As such, the User’s Delivery Pressure may vary and fluctuate depending upon network flow rates, spurline length and configuration of the network from time to time. User’s Delivery Pressure requires to always be capable of meeting the Normal Operating Pressure Range (NOPR, is the range between MinOP and MaxOP for the pipeline network).

Note 2: The User’s delivery temperature shall be the temperature resulting from the delivery pressure requirement described in Note 1 and must always be in the range between the minimum operating temperature and the maximum operating temperature inclusive.